PROGRESS REPORT 1

Graph-Based Spatial-Temporal Analytics for Smart Building Energy Systems

ESHAN Anayed Hossain

ID: 23202807

28 September 2028

Abstract

This report looks into two stages of training a model for detecting components and lines in a Single-Lined Diagram of electrical circuits of a building. Both training and testing was done in Kaggle. The main problem was to detect everything all at once. As a result, the tasks were divided into two parts. First part dives mostly into component detection with was trained on different number of data to check improvements. As the training dataset increased from 1,000 crops across 14 component types to 10,000 crops across 46 types and finally 100,000 crops across 64 types, accuracy on held-out crops increased from 21% to 71% and then 92%. The second part was done using that same model to detect both components and lines in SLDs to get a graph with connecting nodes and edges for the SLD. About 1508 full SLD diagrams were used with 70% trainings, 20% validation and 10% for testing purpose. On the 10% held-out diagrams, the models identified 94.8% of the components without producing false components. It recovered 72.4% of the true connections, while 86.2% of the reported connections were real. All recovered connections were assigned the correct connection type, and the recovered electrical groups achieved a 93.5% overlap with the true groups. Although these results show potential improvements, there might be a chance of overfitting and some of the connections to be missing. This will be looked into in the future tests.

Introduction

Single-Lined Diagrams are used to show the electrical systems of the buildings. They consists of different components such as breakers, distribution boards, transformers and meters, together with the connections between them. Since SLD are mostly given in PDF and images, it is generally difficult to use an automated analysis on these.

One way to make the available information easier to use is to convert the diagram into a graph where components are nodes and the connections are edges. This graph can be later combined with energy data for each components from IoT devices to study and query the whole system and find unusual patterns in the data in future. However, this is more than just detecting symbols and line. There

Data